# ICSI404 – Assignment 3 – Memory and Computer 1

This assignment is extremely important – (nearly) every assignment after this one uses this one!

If you have bugs or missing features in this, you will need to fix them before you can continue to new assignments. This is very typical in software development outside of school.

You must submit .java files. Any other file type will be ignored. Especially “.class” files.

You must not zip or otherwise compress your assignment. Brightspace will allow you to submit multiple files.

You must submit every file for every assignment.

You must submit buildable .java files for credit.

## Introduction

As we have discussed in class, computers have memory based on DRAM. We are going to model that using our Word class. Remember from ICSI333 that memory is accessed through pointers. Pointers, though, are “just” numbers. What are those numbers? They are indices into memory. We will create an array of Words for our memory. But we will also accept a Word as the address. Why? Words hold data. There is no difference to the hardware between a number that is a pointer and a number that is a shoe size! In C, you can do things like this:

*((int *)(0xdff180)) = 0; // set the memory at this address to 0

int newValue = *((int *)(0xdff180)); // get the new value!

A processor is a simple machine. It consists of a simple loop:

while ( not halted)

fetch an instruction from memory

get the data for the instruction (decode)

execute the instruction

store the results

Of course, all of this implies that we have instructions IN memory. How do we get started? In real processors, this is a complex conversation. There is usually a ROM with enough code to read code from storage (disk/SSD).

We will “cheat” this a little. We will have a method in our memory to load a text file into memory that is our program.

We will need a variable to remember what instruction we are working on. That is called “the PC” – the Program Counter.  We will also need a stack (remember this from ICSI333?). The stack is just a pointer into memory. We will call that pointer “SP” – stack pointer. SP and PC are both Words. The PC starts at address 0 in this processor. The stack will start at address 1024. That might seem odd – if our memory is 1024 words, that is addresses 0-1023. Yes, that’s true. But we are defining push as “decrement SP then write to that address”. That means that the first time we push on to our stack, we write to 1024-1 or 1023, the last location in memory.

## Details

Create a new class, MainMemory. It will have three static methods:

public static Word read(Word address)

public static void write(Word address, Word value)

public static void load(String[] data)

MainMemory should have a static array of 1024 words. That number might seem very low (4K), but it will be a lot to manage. Reading should return a new Word object and that writing should not replace the Word object in your memory. Don’t forget to actually allocate Words for your array. Remember that in Java, an array of objects will default to null. Because Java doesn’t have static constructors (boo!), you will have to check to make sure that the memory is constructed before each read and write. For read() and write() you may use getUnsigned() to get the address value as an integer.

Load will process an array of Strings into your simulated DRAM array, starting with 0.

The Strings should be: 32 0’s or 1’s (0 is false, 1 is true)

01101010101010101010010101010101

01101010101010101010010101010101

Etc.

Hint – We will eventually load these from a file. Java’s “files.readAllLines()” is much easier than a Scanner.

Use setBit() to alter each bit in the memory until you run out of lines.

Create a new class – Processor. It will have a PC and an SP, which are Words initialized (you can use “Set()”) as described above. Create a run() method which implements the loop described above. Use empty methods for decode(), execute() and store(). We will implement fetch(). Use a Bit to indicate halted. Your Processor class must not have any data types in it other than Bit and Word.

Fetch() is actually very simple. Get the next instruction from memory (based on the PC). Then increment the PC. Wait – how do we do that? We could use the ALU, but we will need that later for other things. Add increment() to Word. You can do this with only and() and xor() in a loop. The instruction must go into another member Word – “currentInstruction”.

### Testing

We can’t really test our Processor yet. But you can test memory! Use load() to populate memory and then use read() and write() for testing. You must also test Increment().

| Rubric | Poor | OK | Good | Great |
| --- | --- | --- | --- | --- |
| Code Style | Few comments, bad names (0) | Some good naming, some necessary comments (3) | Mostly good naming, most necessary comments (6) | Good naming, non-trivial methods well commented, static only when necessary, private members (10) |
| Unit Tests | None (0) | Partial Coverage (7) | All methods covered, needs more cases (13) | All methods/cases covered (20) |
| Main Memory – load | None (0) | One of: loops over array, converts 0’s and 1’s, populates memory (5) | Two of: loops over array, converts 0’s and 1’s, populates memory (10) | All of: loops over array, converts 0’s and 1’s, populates memory (15) |
| Main Memory – read | None (0) |  | Attempted (5) | Returns a new Word with the same value as the indexed memory (10) |
| Main Memory – write | None (0) |  | Attempted (5) | Alters the indexed memory to have the same value as the Word passed in (10) |
| Processor – setup | None (0) |  |  | Has PC and SP, initialized correctly (5) |
| Processor – run | None (0) | Attempted (5) |  | Checks the halted bit, calls all 4 functions in the proper order (10) |
| Processor – fetch | None (0) | Attempted (5) |  | Fetches the next instruction, calls increment on PC (10) |
| Increment | None (0) | Attempted (5) |  | Correct and uses only xor() and and() (10) |